EL64NA(R100)

Features

◆ Onboard Intel Elkhart Lake series processor;

◆ 2 * Intel i226-V network;

◆ 1 * VGA, 1 * HDMI 2.0, TPM2.0 (optional);

◆ 6 * COM, onboard eMMC optional;

◆ DC 12V, 60W;

◆ Size: 120x120 mm

Specifications

| Processor | Onboard Intel Elkhart Lake series processor |
| --- | --- |
|  | EFI BIOS |
| Memory | 1 * DDR4 SO-DIMM, supports up to 32 GB |
| Display | 1 * VGA, maximum resolution 1920x1200 |
|  | 1 * HDMI2.0, maximum resolution 4K@60Hz |
|  | HDMI+VGA synchronous or asynchronous display |
| Storage | 1 * SATA3.0 |
|  | 1 * M.2 2280 M-Key (PCIe3.0 2x/SATA3.0, hardware selection) |
|  | Onboard eMMC, 32GB/64GB/128GB optional |
| Network | 2 * Intel i226-V network (LAN2 optional Intel i211) |
| Sound card | ALC897 |
| Board edge I/O interface | 1 * VGA, 1 * HDMI |
|  | 2 * USB3.0, 2 * DB9 COM |
|  | 1 * DC JACK, 2 * LAN |
| On-board expansion Interface/function | 0-255 seconds, providing watchdog routine |
|  | Aluminum fanless heatsink (the motherboard has a reserved fan interface that supports automatic and manual fan speed adjustment) |
|  | 4 * USB2.0 pin headers, 2 x 5Pin, pitch 2.0mm |
|  | 6 * COM port (COM1/2 is RS232 /RS485 jumper optional, COM3/4/5/6 is RS232) |
|  | 8-bit GPIO, 2 x 5Pin, pitch 2.0mm |
|  | 1 * Mini PCIe (PCIe + USB2.0 protocol) |
| Power Supply | DC 12V, 60W |
| Working Environment | Working temperature: -20℃ ~ +60℃; Working humidity: 0% ~ 90% relative humidity, no condensation |
|  | Storage temperature: -40℃ ~ +85℃; Storage humidity: 0% ~ 90% relative humidity, no condensation |
| Operating system | Windows10, Windows11, Linux |
| Size | 120x120mm​​ |
| Weight | 260g including heatsink, 130g without heatsink |

EL64NA(R100)

产品尺寸                                                 单位：mm

Ordering Information

| Model | Options | Descriptions |
| --- | --- | --- |
| EL64NA | Onboard Intel Elkhart Lake series processors | J6412/X6413 |
|  | M.2 2280 | PCIe3.0 X2 and SATA3.0 hardware selection |
|  | TPM2.0 | Optional |
|  | COM ports optional | 2COM/6COM optional |
|  | Onboard eMMC | 32GB/64GB/128GB optional |